Lancair	N4XE

# NORMAL PROCEDURES

PILOT'S CHECKLIST

GROSS TAKEOFF WEIGHT 3750 POUNDS

- BEFORE STARTING ENGINE

  - Preflight Inspection – COMPLETE

  - Control Locks – REMOVE

  - Hobbs Time – RECORD (OFF is AFT)

  - Passenger Briefing

    - Seat belts, exit, oxygen masks / use

    - Cell Phones – OFF

  - Oil Cooler Door – CLOSED

  - Autopilot Master Switch – OFF

  - Switches / Lights – AS REQUIRED

  - Landing Gear Switch – DOWN

  - Oxygen Cylinder Reading – CHECK

  - Fuel Selector – FULLEST TANK

  - Throttle – FULL FORWARD

  - Propeller – FULL FORWARD

  - Mixture – FULL RICH

  - Circuit Breakers – IN / ON

  - Batteries and Alternators – ON

  - Hydraulic  Pressure – CHECK

  - Landing Gear Position Indicator Green Lights – ON

  - Annunciator Panel – PRESS TO TEST

  - Brakes – TEST

  - Cabin Pressurization

    - Cabin Pressurization Switch – PRESS or DEPRESS

    - Rate – LEVEL OR MID POSITION

    - Altitude – FIELD ELEVATION

  - Cabin Door – CLOSED, ALL LATCHES ENGAGED, SECURED

  - Door Seal – ACTIVATE, ANNUNCIATOR LIGHT OUT

STARTING ENGINE	2

  - Strobe Lights – ON

  - Auxiliary Fuel Pump - OFF

  - Prime – PRESS 10-15 SECONDS

  - Throttle – OPEN ½ INCH

  - Ignition Switch – BOTH and START

  - Aux Fuel Pump – On if required

  - Throttle – SET to 1000 – 1200 RPM

  - Engine Oil Pressure – CHECK

- BEFORE TAXI

  - Aux Fuel Pump Low Boost – Off

  - Avionics Master Switch – ON

  - Autopilot Master Switch – ON, wait 15 seconds prior to taxi

  - Air Conditioner – AS NEEDED

  - Oil Cooler Door – OPEN WHEN TEMP > 100ºF

  - Fuel Quantity – CHECK

  - Fuel Selector – FULLEST TANK (Right for Left Traffic)

  - EFIS

    - Load and Activate Flight Plan

    - Confirm fuel load

3

- TAXI OR BEFORE TAKEOFF

  - Speed Brakes – OPERATE AND RETRACT

  - Electric Trim – ON, OPERATE AND SET

  - Radios and Avionics – SET

  - SET BAROMETRIC PRESSURE

    - Chelton EFIS

    - Auto Pilot – match Chelton EFIS altitude

    - Dynon – match Chelton EFIS altitude

  - Wing Flaps – SET 10o

  - Xponder switch – CHECK CODE, ON

  - Lights – AS REQUIRED

  - Annunciator Panel – CLEAR

  - Flight Controls – CHECK

  - Engine Runup

    - Check Oil Temp - 83º F MINIMUM

    - Throttles – 1700 RPM

    - Alternators – CHECK VOLTAGE

    - Magnetos – CHECK (150 RPM Maximum Drop With Maximum of 50 RPM Difference)

    - Propeller – EXERCISE (to 1200 RPM)

    - Engine Instruments – CHECK green arc

    - Throttles – 1000 RPM

  - Take Off Briefing – ENGINE OUT, EARLY, LATE, LANDING POSSIBILITIES OFF DEPARTURE END OF RUNWAY

- IMMEDIATELY PRIOR TO TAKEOFF

  - Throttles to 2000 RPM

  - Engine Instruments – CHECK

  - Runway - CHECK

- NORMAL TAKEOFF	4

  - Engine Power – 2700 RPM and 38.5 In. Hg

  - Mixtures – CHECK 41 gal/hr.

  - Engine Instruments – CHECK

  - Elevator Control – RAISE NOSE WHEEL AT 65 KIAS

  - Climb – ESTABLISH 120 KIAS

  - Brakes – TAP TO STOP ROTATION

  - Landing Gear – RETRACT WHEN NO MORE RUNWAY

  - Passing 1000 ft. AGL – RETRACT FLAPS, ACCELERATE TO 165 KIAS, REDUCE THROTTLE TO 31” MAP, REDUCE PROP TO 2500 RPM

G.	AFTER TAKEOFF

- Best Angle of Climb Speed (Vx) –

110 KIAS Flaps at 10º

- Best Rate of Climb Speed (Vy) –

135 KIAS Flaps at 0º

- Normal Climb Power – 2500 RPM and 31.5 In.  Hg

- Normal Climb Speed – 165 KIAS

- Max TIT 1750º F (1850º for 30 Seconds)

- Max CHT 460º

- Cabin Pressurization – SET

- Cabin Altitude Control – SET

- Cabin Rate Control – SET

5

H.	CRUISE

- Engine Speed – 2100 to 2450 RPM

- Throttles – 17 to 31.5 In. Hg

- Mixtures – LEAN (EGT 1550O)

75% Power – 100O rich of max (CHT < 440 O)

65% Power – 75 O rich of max (CHT < 430 O)

55% Power – 50 O rich of max (CHT < 420 O)

- Auxiliary Fuel Pumps – LOW ABOVE 10,000 ft

- Fuel Selectors – AS DESIRED (Feel for Detent), SWITCH TANKS EVERY 30 TO 60 MIN.

- Cabin Pressurization – SET

- Cabin Altitude Control – SET

- Cabin Rate Control – SET

- If Cabin Altitude Annunciator Illuminated (CABIN ALT ABOVE 10,000 FT) DESCEND OR USE SUPPLEMENTARY OXYGEN

I.	LETDOWN
1.	Fuel Selectors – FULLEST TANK

- Auxiliary Fuel Pump – OFF BELOW 10,000 ft.

- Power – AS REQUIRED (Maintain sufficient power for pressurization requirements) (30” Hg.)

- Mixture – ADJUST

- Cabin Pressurization – SET

- Cabin Altitude Control – SET (Field Elevation)

- Cabin Rate Control – SET

- Altimeter – SET

- Flaps - 10º BELOW 174 KIAS

J.	BEFORE LANDING	6
1.	Seat Belts and Shoulder Harness – SECURE

- Fuel Selectors – FULLEST TANK

- Brakes – CHECK FIRM

- Auxiliary Fuel Pump – OFF

- Mixtures – FULL RICH

- Propellers – TO 2500 RPM

- Landing Lights – ON

- Gear – DOWN below 150 KIAS

- Cabin Pressure – ZERO

# K.	AFTER LANDING
1.	Wing Flaps UP

- Hobbs Time – RECORD

- Chilton – Record fuel burn

- Lights – AS REQUIRED

7

# L.	SECURING AIRCRAFT

- Autopilot Master Switch – OFF

- Avionics Master Switch – OFF

- Air Conditioner – OFF

- All Switches except Battery, Alternator, and Magneto Switches – OFF

- Throttle – IDLE

- Propeller – FORWARD

- Timed cool down – CONFIRM

- Mixture – IDLE CUT OFF

- Battery and Alternators – OFF

- Ignition Switch – OFF

- Control Lock(s) – INSTALL

# M.	WEIGHTS

Empty Weight	2396 Pounds

Maximum Gross Weight	3750 Pounds

Payload		1354 Pounds

Full Fuel Load	534

Payload, Full Fuel	820

# N.	FUEL

Tanks	46.5 Gal (279 lbs.) / side usable

89 Gal (534 lbs.) total

# O.	BEST APPROACH – 120 KTS

Level, 10º flaps, 2500 RPM, 11” MAP → 120 Kts

Add gear down → 620 fpm descent

To level off, increase to 19” MAP

# P.	V-SPEEDS KIAS	8

| Vso | Stall speed, landing configuration | 71 |
| --- | --- | --- |
| Vs1 | Stall speed, clean | 80 |
| Vx | Best Angle of Climb | 110 |
| Vy | Best Rate of Climb | 135 |
|  | Best Angle of Glide | 120 |
| Vlo | Maximum Gear Extension Speed | 150 |
| Vlo | Maximum Gear Retraction Speed | 120 |
| Vle | Maximum Gear Extended Speed | 165 |
| Vfe | Max. Flap Operating Speed, 0-10º | 174 |
| Vfe | Max. Flap Operating Speed, 10º-Max | 132 |
| Va | Design maneuvering speed | 170 |
| Vno | Maximum Design Cruising Speed | 220 |
| Vne | Never-Exceed Speed | 274 |
|  | Max demonstrated x-wind component | 25 |

| White Arc | 71–132 |
| --- | --- |
| Green Arc | 80–220 |
| Yellow Arc | 220–274 |
| Red Radial | 274 |

- OIL TEMPERATURE ºF

| Maximum | 220 |
| --- | --- |
| Caution Range | 200 – 220 |
| Operating Range | 160 – 200 |
| Takeoff Minimum | 100 |

- OIL PRESSURE P.S.I.

| Maximum | 105 |
| --- | --- |
| Caution Range | 10 – 45, 85 – 105 |
| Normal Range | 45 – 85 |
| Minimum | 25 |

- FUEL PRESSURE P.S.I.

| Maximum | 75 |
| --- | --- |
| Caution Range | <10,>35 |
| Normal Range | 10 – 35 |
| Minimum | 7 |