Hot Start Procedures

Option 1 
· All knobs (throttle, prop, mixture) full in, 
· touch the fuel primer briefly to register a small squirt on the fuel flow guage (emphasis on briefly), 
· and crank the starter. 
· Engine will start and pull back on throttle to adjust.  
· Option to temporarily pull back a bit on mixture (titrate to starting) to help further lean the injection.
Option 2


· I set the throttle to 3/8" in (just a bit more in than for a cold start), 
· put the mixture to full rich, 
· low boost pump on and start cranking immediately. 
· If engine dies: keep cranking + 2-second shot of high boost--and no more!--leaving the low boost still on. 
· If it doesn't start within 10-15 seconds, turn off the boost pumps and stop cranking.  Let it sit with the throttle full open for a couple of minutes...don't rush it...and try the same procedure again. 
· When the engine starts, it will accelerate to 1200 RPM or so. Throttle back, but not to idle, as it will likely stall at idle until the lines/galleries are purged of air/vapor.
Option 3 
· Wide open throttle
· Fill rich mixture
· Low boost on for 6 seconds...then boost off Attempt start with one finger ready to turn on low boost and your right hand on the throttle ready to retard when engine fires off When engine does fire off, immediately turn low boost on and retard throttle
Option 4

· Wide open throttle, mixture full rich

· high boost on for a few seconds.  Just long enough to fill the lines with fuel. If fuel runs out the sniffle valve the high boost was on too long.

· Leave throttle near wide open and pull mixture to idle cut off.

· Turn low boost on and immediately start cranking engine while pushing in the mixture knob slowing until engine fires. 

· Be prepared to retard the throttle as the engine speed will go quite high if you get a clean start.  Leave the low boost on for several minutes and control idle quality with the mixture knob. Once the idle quality is good, the low boost electric fuel pump can be turned off.

Option 5 (works best for my engine)
· Mixture cut off, throttle closed, high boost for 30 sec.

· Fuel pump on low boost
· Throttle normal idle, mixture cut off

· Start cranking with on hand while VERY SLOWLY richening the mixture with other hand
· Engine starts at ideal mixture for existing conditions – stop cranking

· Continue richening as idle speed increases to normal idle speed – not richer until you are ready to increase throttle
